# Brakelight

GT Off-Road Racing | Data Acquisition

Param Pithadia

2/16/2023

Table of Contents

1.0 Overview	2

1.1 Introduction	2

1.2 Point of Contact	2

2.0 Hardware Reference	3

2.1 Circuit	3

2.1.1 Wiring	3

3.0 Software Theory of Operation	4

4.0 References	5

5.0 Revision History	6

# Overview

## Introduction

We use one red brake light (red LED and red lens) on the back of the car, connected to the brakes through two hydraulic brake light switches.

## Point of Contact

Ryan Chen (rchen390@gatech.edu): Created documentation standard

# Hardware Reference

## Circuit

The brake light will turn on if pressure is detected in either switch, closing the switch and completing the path between the battery and the light, that is why the two brake switches are wired in parallel, and the battery and light are in series. For installing the brake switches, please talk to the breaks lead.

We can also include two brake pressure sensors, but that is up to the discretion of the breaks lead.

### Wiring

When wiring the kill switch, there are some important things to note. The red wire is vcc, the white wire is gnd, and the black wire we do not use so you can just fold it back and cover it with heat shrink.

When crimping the connectors on, after stripping the wires slide the green rubber covers onto the wire, align the pin’s largest back ring with the rubber cover, and then use the crimping tool to crimp.

Then you can just push the pins into their housing. Remember that male pins go with female housing. Refer to the wiring standards document to confirm which pin is gnd and which is vcc. When pushing both pins into their housing, make sure that they are in the same orientation to properly align the pins in the center of the housing and protect them from breaking.

# Software Theory of Operation

# References

# Revision History

1/7/2021 (Ryan Chen) – This is an example of a revision comment. Removed parts list from references and added Excel template for Bill of Materials. Removed parts list from references and added Excel template for Bill of Materials.

2/22/2021 (Ryan Chen) – Updated automatic lists for template.

2/16/2023 (Param Pithadia) – Added circuit section and hyperlinks for parts.