# Wiring and Connector Standards

GT Off-Road Racing | Data Acquisition

Ryan Chen

06/14/2021

Table of Contents

1.0 Overview	2

1.1 Introduction	2

1.2 Point of Contact	2

2.0 Wiring Standards	3

2.1 Connector genders between electrical components	3

2.2 Securing wires	3

3.0 Connector Pinout Standards	4

3.1 Introduction	4

3.2 XT30	4

Power	4

3.3 Micro-Fit	5

2-pin	5

3-pin	5

Analog Signal	5

Other	5

4-pin	5

I2C	5

Serial (RX/TX)	5

Other	6

6-pin	6

SPI	6

3.4 Weather-Pack	7

Power	7

4.0 References	8

5.0 Revision History	9

# 1.0 Overview

## 1.1 Introduction

# 2.0 Wiring Standards

## 2.1 Connector genders between electrical components

All connectors attached to an electrical component/system (PCB, battery, sensor, etc.) from components are male.

[Add analogy about islands and bridges]

All bridging cables/wires are female.

Figure 1: High-level overview of electrical wiring

## 2.2 Securing wires

Wires are secured to the car by zip ties. The zip ties typically go through mounting tabs on the car, but also go around the chassis frame.

# 3.0 Connector Pinout Standards

## 3.1 Introduction

A few key rules:

- If a custom color/pin pattern is used, include the connector diagram and pinout table in the relevant project’s documentation.

## 3.2 XT30

Figure 2: XT30 Connectors

#### Power

| Pin | Color | Description |
| --- | --- | --- |
| + | Red | Power |
| - | Black | Ground |

## 3.3 Micro-Fit

### 2-pin

### 3-pin

Figure 3: Micro-Fit 3-pin connector

| #### Analog Signal Pin; Color; Description. 1; Red; Power. 2; White; Analog signal. 3; Black; Ground. | #### Digital Signal Pin; Color; Description. 1; Red; Power. 2; White; Digital signal. 3; Black; Ground. |
| --- | --- |
| #### Other Pin; Color; Description. 1; Red. 2; White. 3; Black. |  |

### 4-pin

Figure 4: Micro-Fit 4-pin connector

| #### I2C Pin; Color; Description. 1; Red; Power. 2; Green; I2C Data (SDA). 3; White; I2C Clock (SCL). 4; Black; Ground. | #### Serial (RX/TX) Pin; Color; Description. 1; Red; Power. 2; Green; RX. 3; White; TX. 4; Black; Ground. |
| --- | --- |
| #### Load Cell Pin; Color; Description. 1; Red; Power. 2; Green; A+. 3; White; A-. 4; Black; Ground. | #### Load Cell Signal Pin; Color; Description. 1; Red; Power. 2; Green; VOUT. 3. 4; Black; Ground. |
| #### Other Pin; Color; Description. 1. 2. 3. 4. |  |

### 6-pin

Figure 5: Micro-Fit 6-pin connector

| #### SPI Pin; Color; Description. 1; Red; Power. 2; Green; MOSI/SDI. 3; Blue; MISO/SDO. 4; Orange; Serial clock (SCK). 5; White; Chip select (CS). 6; Black; Ground. | Pin; Color; Description. 1; Power. 2. 3. 4. 5. 6; Ground. |
| --- | --- |

## 3.4 Weather-Pack

Figure 6: Weather-Pack 2-pin connector

#### Li-Po Batteries

| Pin | Color | Description |
| --- | --- | --- |
| 1 | Black | Ground |
| 2 | Red | Power |

# 4.0 References

# 5.0 Revision History

9/16/2021 (Ryan Chen) – Created high-level wiring standards for electrical systems/PCBs on the car. Inserted pictures of pins and empty tables of pinout diagrams for connectors.