# Wiring and Connector Standards

GT Off-Road Racing | Data Acquisition

Ryan Chen

06/14/2021

Table of Contents

1.0	Overview	3

1.1	Introduction	3

2.0	Wiring Standards	4

2.1	Connector genders between electrical components	4

2.2	Securing wires	4

3.0	Connector Pinout Standards	5

3.1	Introduction	5

3.2	XT30	5

Power	5

3.3	Molex Micro-Fit 3.0	6

3.3.1	2-pin	6

Other	6

3.3.2	3-pin	6

Signal	6

Other	6

Serial or UART (for self-powered systems)	6

3.3.3	4-pin	7

I2C	7

Serial/UART (for downstream devices that require a power source)	7

Strain Gauge PCB Input	7

Strain Gauge PCB Output Signal	7

Aux DAQ Serial (RX/TX)	7

Dashboard Serial (RX/TX)	7

Other	7

3.3.4	6-pin	8

SPI	8

Other	8

3.4	Weather-Pack	9

3.4.1	2-pin	9

Li-Po Batteries	9

3.5	Molex Squba	9

3.5.1	3-pin	9

I2C	9

3.5.2	4-pin	9

I2C	10

3.6	Metri-Pack	10

3.6.1	3-pin	10

Signal	10

4.0	References	12

5.0	Revision History	13

# Overview

## Introduction

# Wiring Standards

## Connector genders between electrical components

All connectors attached to an electrical component/system (PCB, battery, sensor, etc.) from components are male.

[Add analogy about islands and bridges]

All bridging cables/wires are female.

Figure 1: High-level overview of electrical wiring

## Securing wires

Wires are secured to the car by zip ties. The zip ties typically go through mounting tabs on the car, but can also go around the chassis frame.

# Connector Pinout Standards

## Introduction

A few key rules:

- If a custom color/pin pattern is used, include the connector diagram and pinout table in the relevant project’s documentation.

## XT30

| Female | Male |
| --- | --- |

Figure 2: XT30 Connectors

#### Power

| Pin | Color | Description |
| --- | --- | --- |
| + | Red | Power |
| - | Black | Ground |

## Molex Micro-Fit 3.0

### 2-pin

| Female | Male |
| --- | --- |

Figure 3: Micro-Fit 2-pin connector

| #### Other Pin; Color; Description. 1; Red. 2; White; Ground. |  |
| --- | --- |

### 3-pin

| Female | Male |
| --- | --- |

Figure 4: Micro-Fit 3-pin connector

| #### Signal Pin; Color; Description. 1; Red; Power. 2; White; Signal. 3; Black; Ground. | #### Other Pin; Color; Description. 1; Red. 2; White. 3; Black. |
| --- | --- |
| #### Serial or UART (for self-powered systems) Pin; Color; Description. 1; Red; TX or RX. 2; White; TX or RX. 3; Black; Ground. |  |

### 4-pin

| Female | Male |
| --- | --- |

Figure 5: Micro-Fit 4-pin connector

| #### I2C Pin; Color; Description. 1; Red; Power. 2; Green; I2C Data (SDA). 3; White; I2C Clock (SCL). 4; Black; Ground. | #### Serial/UART (for downstream devices that require a power source) Pin; Color; Description. 1; Red; Power. 2; Green; RX or TX. 3; White; RX or TX. 4; Black; Ground. |
| --- | --- |
| #### Strain Gauge PCB Input Pin; Color; Description. 1; Red; Power. 2; Green; A+. 3; White; A-. 4; Black; Ground. | #### Strain Gauge PCB Output Signal Pin; Color; Description. 1; Red; Power. 2; Green; Vout. 3; N/A; N/A. 4; Black; Ground. |
| #### Aux DAQ Serial (RX/TX) Pin; Color; Description. 1; Red. 2; N/A. 3; White. 4; Black; Ground. | #### Dashboard Serial (RX/TX) Pin; Color; Description. 1; Red; Aux DAQ TX. 2; N/A. 3; White; Aux DAQ RX. 4; Black; Ground. |
| #### Other Pin; Color; Description. 1; Red; Power. 2; Green. 3; White. 4; Black; Ground. |  |

### 6-pin

| Female | Male |
| --- | --- |

Figure 6: Micro-Fit 6-pin connector

| #### SPI Pin; Color; Description. 1; Red; Power. 2; Green; MOSI. 3; Blue; MISO. 4; Orange; Serial clock (SCK). 5; White; Chip select (CS). 6; Black; Ground. | #### Other Pin; Color; Description. 1; Red; Power. 2; Green. 3; Blue. 4; Orange. 5; White. 6; Black; Ground. |
| --- | --- |

## Weather-Pack

### 2-pin

| Female | Male |
| --- | --- |

Figure 7: Weather-Pack 2-pin connector

Li-Po Batteries

| Pin | Color | Description |
| --- | --- | --- |
| 1 | Black | Ground |
| 2 | Red | Power |

## Molex Squba

### 3-pin

| Female | Male |
| --- | --- |

Figure 8: Molex Squba 4-pin connector

| #### I2C Pin; Color; Description. 1; Red; Power. 2; White; Signal. 3; Black; Ground. 4; N/A; Plugged shut (waterproof). |  |
| --- | --- |

### 4-pin

| Female | Male |
| --- | --- |

Figure 9: Molex Squba 4-pin connector

| #### I2C Pin; Color; Description. 1; Red; Power. 2; Green; I2C Data (SDA). 3; White; I2C Clock (SCL). 4; Black; Ground. |  |
| --- | --- |

## Metri-Pack

### 3-pin

Female

Figure 10: Metri-Pack 3-pin connector

#### Signal Pin; Color; Description. A; Red; Power. B; Black; Ground. C; White; Signal.

# References

# Revision History

9/16/2021 (Ryan Chen) – Created high-level wiring standards for electrical systems/PCBs on the car. Inserted pictures of pins and empty tables of pinout diagrams for connectors.

5/15/2022 (Ryan Chen) – Added Squba and Metri-Pack information